BUCKLEYS ROAD WASTE MANAGEMENT FACILITY

SITE PROCEDURE FOR THE SAFE HANDLING AND DISPOSAL OF ASBESTOS

The Buckleys road waste management facility is a Class 2 Category 64 facility licensed by DWER to accept asbestos waste.

The procedure for acceptance of the asbestos is as follows:

- No asbestos will be accepted unless it complies with all legislative requirements, environmental protection (controlled waste) regulations 2004 and the guideline for controlled waste treatment for disposal sites by the Department of Water, Environment and Regulation.

- On inquiry for disposal of asbestos, this procedure and check list will be Emailed or handed to the inquirer to read and complete, then return to the facility in person or by Email to alan.batt@broome.wa.gov.au once completed.

- All asbestos request is required to be booked in at least 2 hours prior to disposal to ensure we have manpower and machinery available to cover. Customers will be required to sign a declaration at time of entry to the facility.

- When the demolition involves more than 5 cubic meters the WMF office needs to be contacted at least a week prior to delivery so that all the necessary arrangements can be made, and paperwork completed.

- Asbestos will only be accepted if it is double wrapped in black plastic and labelled correctly if it is intended to be transported or disposed of in a different way then the facility needs to be informed and all approvals for this type of disposal needs to be forwarded to the facility for approval before any disposals can take place.

- Asbestos in commercial quantities will only be accepted if the asbestos is correctly wrapped and labelled and can be unloaded by crane or forklift and stacked in a designated area for contaminated waste.

- Private customers and all fencing and building contractors must present asbestos sheeting in flat packs that conform with all Regulations. The flat packs must be able to be transferred by the customer into the site loader or the hazardous waste cell.

- Site employees are not to assist customers with the unloading of asbestos unless absolutely necessary. However, must observe the unloading to ensure no damage to the packs occur. Any damage must be repaired immediately.

| # Waste Management Facility # Asbestos Acceptance Checklist |  |
| --- | --- |
| # All asbestos waste must be booked in at least 2hrs prior to delivery to the facility. When the demolition involves more than a cubic meter then the WMF office needs to be contacted at least a week prior to delivery so that all the necessary arrangements can be made. All |  |
| # Date of Delivery |  |
| # Waste Producer / Demolition Contractor |  |
| # Contact Details |  |
| # Type of Asbestos |  |
| # Amount e.g. number and size of loads |  |
| # Source of Waste |  |
| Waste Carrier |  |
| # Vehicle Registration number |  |
| # Load received by (Name and Sign) |  |

I ____________________________ declare that I have read the procedure processes, understand and will abide by them when disposing of asbestos or asbestos containing material. Signed: ____________________________