BUCKLEYS ROAD WASTE MANAGEMENT FACILITY

SITE PROCEDURE FOR THE DISPOSAL OF CONSTRUCTION & DEMOLITION WASTE AT THE RESOURCE RECOVERY FACILITY LOCATED AT LOT 400 BUCKLEYS ROAD

The Lot 400 Buckleys road resource recovery facility is licensed by DWER to accept resource recoverable materials e.g. green waste, steel, timber, glass, used tyres, concrete, bricks & pavers. For C&D waste to be classified as recoverable and attract the cheaper price for clean materials, it must not be contaminated by foreign material, less than 5% by volume per load, or it will be classified as mixed waste and attract the higher price.

Low Risk or High Risk

The risk of a load containing asbestos is related to the type and source of the waste. In general, buildings and structures constructed after 1990 are unlikely to have asbestos containing materials within them, whereas buildings and structures constructed before this date may have been built using asbestos containing materials.

Where the source of the load can clearly be determined to be a building or structure constructed after 1990 then the load can be considered to represent a low risk of asbestos contamination and managed as outlined in the following section. Where the waste originates from a building constructed before 1990 or there is uncertainty over this issue, the risks associated with asbestos in the load must be established in line with the Risk Classification Matrix below.

Where the incoming waste is mixed, the risk of asbestos contamination is high.

| Risk Classification Matrix |  |  |  |
| --- | --- | --- | --- |
|  | Type of load |  |  |
| Material Type | Commercial | Public, utes, cars and trailers* | Skip bins |
| Clean Concrete (without formwork) | Low | High | High |
| Clean Brick Low | Low | High | High |
| Clean Bitumen / Asphalt | Low | High | High |
| Mixed Construction waste | High | High | High |
| Mixed Demolition waste | High | High | High |

* if it is possible to view the entire load of incoming C & D material (eg a small trailer with a shallow load, then consideration may be given to classifying these loads as low risk (Risk Matrix Classification adapted from WorkSafe Victoria 2006 and WMAA 2009)

| # C&D Resource Recovery Site Waste Acceptance Checklist |  |
| --- | --- |
| # Date |  |
| # Waste Producer |  |
| # Contact Details |  |
| # Waste Carrier |  |
| # Vehicle Registration number |  |
| # Source of Waste & Age of structure (if known) |  |
| # Is the load ‘Low Risk’ or ‘High Risk’ See information over the page | # Low Risk / High Risk |
| # Is the waste contained on the load consistent with the site licence? |  |
| # Is all the waste entirely free of asbestos? |  |
| # If the waste contains or is suspected to contain asbestos then do not accept the waste at the Stockpile Recycling Site. The load shall be directed to the landfill for safe disposal and this checklist will be kept at the gatehouse for future reference. |  |
| # Load received by (Name and Sign) |  |

I ______________ declare that this load is free from asbestos or asbestos containing material. Signed: ____________________________